Energy Plus Savings Estimator

Technical Specification — Variables, Assumptions, and the Complete Calculation Model

Reference build: ep-calc-v18.html · United States, Canada, México · This document captures everything needed to rebuild the calculator from scratch.

# 1. Purpose and design philosophy

The estimator turns a single input — a facility’s annual electricity spend — into a conservative, defensible range of annual savings, then defers the precise figure to a human "22-Minute Facility Review." Every modelling choice below follows from a small set of presuppositions. If you rebuild the calculator, honor these first; the numbers are downstream of them.

## Core presuppositions

- Percentage-of-bill model. Every savings stream is computed as a fraction of the user’s entered annual spend. The absolute electricity rate ($/kWh) is therefore never an input — it is already baked into the bill. This makes the model currency- and country-neutral at its core.

- A range, not a point. Each stream produces a floor and a ceiling. The displayed result is floor → ceiling. The floor is deliberately conservative; the ceiling is the optimistic-but-plausible bound.

- Understate by design. Wherever a choice exists, the model picks the assumption that lowers the estimate (low end of count buckets, conservative stream floors). The stated goal: the verified number should beat the estimate.

- Power harmonization is universal. The harmonization (power-quality) floor is physics, identical in any country, and is always included. Cooling, demand, lighting, motors, heating, rate negotiation, and power factor are conditional streams.

- Credibility before persuasion. The front-end cites the engineering standards the work meets (IEEE 519 power quality, ASHRAE HVAC, ISO). Consumption-statistics agencies (DOE/EIA in the US, NRCan/StatCan in Canada, CONUEE in México) are cited only as the calibration source for the percentage shares.

# 2. The savings model (compute)

Let S = annual electricity spend. Two pools are derived first, then each stream draws from them. All stream outputs are dollars per year.

## 2.1 Pools

harmo  = harmoRate × S                    // harmoRate = 0.09 (always included)

H      = hvacShare × S                     // hvacShare = 0.40  (HVAC pool)

coolEff = (heating is GAS) ? 1.0 : coolShare

C      = H × coolEff                        // cooling pool

He     = H × (1 − coolEff)                  // heating pool

coolShare = max(stateCool, coolMin) / 100   // region cooling %, floored by facility

Gas heating sets coolEff = 1.0, so the entire electric HVAC pool is treated as cooling and the heating pool is zero. Electric heating splits the pool by coolShare.

## 2.2 Streams (floor / ceiling)

| Stream | Floor | Ceiling | Condition |
| --- | --- | --- | --- |
| Harmonization | 0.09 × S | 0.09 × S | always |
| Cooling | C × 0.15 | C × 0.55 | tCool on |
| Heating — known resistance | 0 | He × 0.80 | tHeat on |
| Heating — known heat pump | 0 | He × 0.50 | tHeat on |
| Heating — "not sure" (bracket) | He × 0.50 | He × 0.80 | tHeat on (default) |
| Demand reduction | 0.05 × demPortion × S | battery? demPortion×S : floor | tDem on |
| Lighting (outdoor/off-grid) | OUTDOORf × S × litMult | OUTDOORc × S × litMult | tLit on |
| Rate negotiation | 0.025 × S | 0.06 × S | tNego on (deregulated) |
| Smart motor start | ms × demPortion × S × 0.20 | ms × demPortion × S × 0.50 | tMot on |
| Power factor | pfShare × S × 0.60 | pfShare × S × 0.90 | MX or CA, PF active |

litMult = 1.0 for off-grid solar lighting, 0.88 for grid-tied. COOL_FLOOR = 0.15, COOL_CEIL = 0.55. HEAT.heatpump = 0.50, HEAT.resistance = 0.80.

## 2.3 Combining demand and totals

demPool      = demPortion × S

combDemFloor = demFloor + motFloor

combDemCeil  = min(demCeil + motCeil, demPool)   // capped at the demand pool

TOTAL floor  = nego_f + harmo + cool_f + combDemFloor + lit_f + pf_f + heat_f

TOTAL ceil   = nego_c + harmo + cool_c + heat_c     + combDemCeil + lit_c + pf_c

The percentage shown ("X% to Y% of your annual bill") is TOTAL_floor/S to TOTAL_ceil/S.

# 3. Heating model (COP logic)

Heating savings equal the heating pool He multiplied by an efficiency-gain fraction derived from the coefficient of performance (COP): savings fraction = 1 − COP_baseline / COP_new. The two constants encode this:

- HEAT.resistance = 0.80 — replacing electric resistance (COP ≈ 1) yields ~80% heating-energy reduction.

- HEAT.heatpump = 0.50 — replacing an efficient COP-3 heat pump yields ~50% (1 − 3/6).

The Stage-3 question "How does your facility heat today?" sets heatComp / tHeat:

| Answer | heatComp | tHeat | Effect on math |
| --- | --- | --- | --- |
| Natural gas | gas | off | coolEff = 1.0 (all HVAC = cooling); no heating savings |
| Heat pump | heatpump | on | ceiling only: He × 0.50 |
| Electric resistance | resistance | on | ceiling only: He × 0.80 |
| I’m not sure (default) | resistance | on | BRACKET: floor He × 0.50, ceil He × 0.80 |

Rationale: a facility on gas would know (separate bill), so "not sure" assumes electric heat and brackets the savings between an efficient heat pump (low) and resistance (high). He is climate-weighted through coolShare, so cold regions get a larger heating bracket automatically.

# 4. Facility count, portfolio, and qualification

## 4.1 Count buckets

The user picks how many similar facilities they operate. Each bucket carries a low and high end; the low end drives qualification, the [low, high] span drives the portfolio range.

| Bucket | Low | High |
| --- | --- | --- |
| Just one for now | 1 | 1 |
| 2 to 4 | 2 | 4 |
| 5 to 9 | 5 | 9 |
| 10 to 19 | 10 | 19 |
| 20 or more | 20 | 20 |

## 4.2 Spend tiers and turnkey qualification

The qualifying driver is per-facility MONTHLY spend = S / 12. The lower the monthly spend, the more sites are needed for full turnkey ("done for you") to pay off.

| Per-facility monthly spend | Min sites for turnkey | State if below min |
| --- | --- | --- |
| Under $1,500 | never (equipment-only) | equipment_only |
| $1,500 – $1,999 | 20 | turnkey_warm |
| $2,000 – $3,499 | 10 | turnkey_warm |
| $3,500 – $4,999 | 5 | turnkey_warm |
| $5,000 and up | 1 | turnkey_warm |

Three states result: equipment_only (under $1,500 — self-install messaging, DIY SMS CTA, no turnkey language); turnkey_fit (spend qualifies AND count ≥ min); turnkey_warm (spend qualifies but count below min — full turnkey kept, candor note added). $1,500/mo is the only hard gate.

## 4.3 Portfolio range

portfolioFloor = perFacilityFloor × lowEndCount

portfolioCeil  = perFacilityCeil  × highEndCount

// e.g. "10 to 19": floor × 10  →  ceil × 19

When count > 1, the result shows the portfolio total as the hero figure and the per-facility figure as a secondary block. Multi-state portfolios are flagged for a tailored review (no per-state portfolio math in this version).

# 5. Escalation and the 5/10-year projection

escalFactor(rate, years) = ((1 + rate)^years − 1) / rate     // compounding annuity sum

cum-5yr  = heroSavings × escalFactor(rate, 5)

cum-10yr = heroSavings × escalFactor(rate, 10)

rate is the region’s annual electricity-rate escalation, looked up by tier. Figures ≥ $1,000,000 render as $X.XXXm, ≥ $1,000 as $Xk.

| Tier | Range | Rate | Applies to |
| --- | --- | --- | --- |
| 1 | 4–6% | 0.045 | CA*, HI, MA, NY, NJ, CT (US high-cost) |
| 2 | 3–4% | 0.035 | MD, DC, IL, PA, TX, VA, RI, NH, ME, DE, VT |
| 3 | 2–3% | 0.025 | default for unlisted US states |
| 4 | 1–2% | 0.018 | WA, ID, MT, ND, SD, NE, WY |
| 5 | volatile | 0.025 | Puerto Rico |
| 6 | 3–5% | 0.035 | México (all CFE regions) |
| ca-on | ~3.5% | 0.035 | Ontario |
| ca-qc | ~2.8% | 0.028 | Québec |
| ca-mb | ~2.8% | 0.028 | Manitoba |
| ca-sk | ~2.0% | 0.020 | Saskatchewan |

*CA = California here, not Canada. Canadian provinces use the ca-* keys. Escalation only affects the projection boxes, not the headline savings.

# 6. Global parameters and constants

| Parameter | Default | Meaning |
| --- | --- | --- |
| hvacShare | 0.40 | HVAC share of the total electric bill |
| harmoRate | 0.09 | Harmonization (power-quality) floor, share of bill |
| coolMin | 0 | Facility refrigeration floor for cooling share |
| stateCool | set by region | Cooling % of HVAC for the region |
| demPortion | 0.40 | Demand-charge share of bill (engine default) |
| demRed | 0.05 | Demand-reduction floor fraction |
| demRate | 12 | $ per kW (display-only, implied-kW readout) |
| demBattery | false | If true, demand ceiling = full demand pool |
| litMode | offgrid | offgrid → litMult 1.0; grid → 0.88 |
| COOL_FLOOR / COOL_CEIL | 0.15 / 0.55 | Cooling savings rate bounds |
| HEAT.heatpump / .resistance | 0.50 / 0.80 | Heating savings rate by baseline |
| nego floor / ceil | 0.025 / 0.06 | Rate-negotiation bounds (deregulated) |
| motor floor / ceil | 0.20 / 0.50 | Smart-motor multipliers |
| PF floor / ceil | 0.60 / 0.90 | Power-factor recovery bounds |
| mxPFShare / caPFShare | 0.08 / 0.05 | PF share of bill (MX / CA) |

## 6.1 Default stream toggles

tCool = on, tHeat = on, tDem = on, tLit = on, tMot = off, tNego = off. heatComp = resistance, heatChoice = "I’m not sure". Country default = US, language = English.

# 7. Per-facility data tables

Sixteen facility types: ind (industrial/mfg), datactr (data center), health (healthcare), cold (cold storage), research (lab/campus), super (supermarket), edu (education), whse (warehouse), hotel, multi (multifamily), office, fitness, retail, qsr (quick-serve), cstore (convenience), mixed. The math-relevant per-facility tables follow.

## 7.1 Outdoor / off-grid lighting share (OUTDOOR: floor f, ceil c)

| Facility | f (floor) | c (ceil) |
| --- | --- | --- |
| cold | 0.01 | 0.03 |
| super | 0.02 | 0.05 |
| cstore | 0.08 | 0.15 |
| hotel | 0.04 | 0.09 |
| health | 0.03 | 0.07 |
| fitness | 0.03 | 0.07 |
| edu | 0.05 | 0.10 |
| multi | 0.05 | 0.10 |
| qsr | 0.04 | 0.08 |
| office | 0.05 | 0.10 |
| retail | 0.06 | 0.12 |
| mixed | 0.04 | 0.08 |
| whse | 0.06 | 0.12 |
| ind | 0.03 | 0.07 |

## 7.2 Motor share (MOTOR.ms) — eligible facilities only

Smart-motor stream is offered only for ind, cold, whse, health, and only when annual spend ≥ $150,000.

| Facility | ms |
| --- | --- |
| ind | 0.55 |
| cold | 0.50 |
| whse | 0.40 |
| health | 0.30 |

## 7.3 Square-footage rate (SQFT_RATE, $/sq ft) — used to estimate floor area for the US Section 179D step

| Facility | $/sq ft | Facility | $/sq ft |
| --- | --- | --- | --- |
| cold | 3.20 | office | 2.80 |
| super | 8.00 | retail | 2.60 |
| cstore | 10.00 | mixed | 2.00 |
| hotel | 3.50 | whse | 1.00 |
| health | 4.50 | ind | 4.50 |
| fitness | 3.20 | edu | 2.50 |
| qsr | 5.50 | multi | 1.40 |

estimateSqft(key, S) = round(S / rate) snapped to 1000/500/100. Section 179D deduction shown up to $5.94/sq ft (US only).

# 8. Country-specific behavior

## 8.1 United States

- Location: 50 states + Puerto Rico. Each state has a cooling % (STATE_COOL) and an escalation tier (ESC_TIER).

- Rate-negotiation stream ON for deregulated states (DEREG_STATES: CT, DE, IL, ME, MD, MA, MI, NH, NJ, NY, OH, PA, RI, TX, DC). OFF for regulated states and Puerto Rico.

- Stage 3 = Section 179D deduction (sq-ft slider). Currency USD. Language English.

STATE_COOL (cooling % of HVAC): AZ/TX/FL/LA/HI/PR 80; AL/AR/GA/KY/MS/NV/NM/NC/OK/SC/TN 65; CA/DE/DC/KS/MO/OR/VA/WA/WV/MD 50; CO/CT/IL/IN/IA/MA/MI*/NE/NJ/NY/OH/PA/RI/UT 35; AK/ID/ME/MN/MT/NH/ND/SD/VT/WI/WY 22. (Lower = colder/more heating.)

## 8.2 México

- Location: 6 CFE regions. Each sets cooling %, electric-heating flag, demand share, and a description. Tariff (PDBT/GDMTO/GDMTH) sets demand share; auto-detected from spend and facility type.

- Power-factor stream ON (mxPFShare 0.08) — CFE penalizes low PF. Rate negotiation OFF (CFE is sole supplier).

- Currency shown in USD with a peso equivalent line (mxFxRate, default 17.50 MXN/USD). Language Spanish. No Section 179D.

| CFE region | cool % | elec heat | demand % |
| --- | --- | --- | --- |
| Norte | 68 | yes | 0.42 |
| Occidente | 60 | no | 0.38 |
| Central | 45 | yes | 0.35 |
| Centro-Norte | 55 | no | 0.38 |
| Oriente | 75 | no | 0.40 |
| Sur-Sureste | 82 | no | 0.42 |

Tariff demand shares: PDBT 0.25, GDMTO 0.35, GDMTH 0.45.

## 8.3 Canada (launch: Ontario, Québec, Manitoba, Saskatchewan)

- Phased rollout: only the four launch provinces run the calculator. Other provinces/territories show an "expanding soon" message and a capture CTA; no estimate is produced.

- Power-factor stream ON for all four (caPFShare 0.05): ON/QC penalize below 0.90 PF; MB/SK bill demand on kVA (continuous PF incentive). Rate negotiation ON for Ontario (competitive market) only.

- Cooling share is the editable auto-assumption (defers to a facility’s refrigeration floor). Currency CAD. Language English (French deferred). Stage 3 = CCA 43.1/43.2 + Clean-Tech-ITC note instead of Section 179D.

| Province | cool % | escalation | market (nego) | PF |
| --- | --- | --- | --- | --- |
| Ontario | 30 | 3.5% | competitive (on) | yes |
| Québec | 22 | 2.8% | regulated (off) | yes |
| Manitoba | 22 | 2.8% | regulated (off) | yes |
| Saskatchewan | 22 | 2.0% | regulated (off) | yes |

Cooling shares are calibrated to the US STATE_COOL scale (floor 22 for cold climates; southern Ontario 30), confirmed against ECCC degree-day data, and editable in-app.

# 9. User flow and stages

- Stage 0 — Country: choose US / Canada / México (flag cards; Canada flagged "4 provinces").

- Stage 1 — Facility type: pick one of the 16 types (sets per-facility cooling floor, lighting, motor eligibility, copy).

- Stage 2 — Inputs (progressive disclosure): (1) monthly spend per facility via a two-step band selector; then (2) facility count chips appear; then (3) location (state / province / CFE region) appears. A live signal box explains turnkey vs equipment-only fit.

- Stage 3 — Final question: square footage / 179D (US), or CCA-ITC note (Canada), or nothing extra (México); then "How do you heat today?" (sets the heating model).

- Result: animated reveal of the floor→ceiling range, % of bill, 5/10-year projection, per-facility vs portfolio, narrative, and CTA (turnkey booking + SMS, or equipment-only DIY SMS).

## 9.1 Result-summary object (for downstream capture)

On result, state.resultSummary exposes: state (equipment_only / turnkey_fit / turnkey_warm), countLabel, lowEndCount, highEndCount, spendTier, perFacilityMonthly, singleFacilityFloor/Ceil, portfolioFloor/Ceil — ready for a HubSpot-style capture step.

# 10. Data confidence and rebuild notes

- US STATE_COOL and ESC tiers are internal calibrations to DOE/EIA building data. Mexico CFE regions/tariffs are modeled estimates. Canada launch-province data: market structure and PF penalties are tariff-verified (high confidence); cooling shares are US-scale calibrations confirmed by ECCC degree days; escalation figures are primary-source utility rate histories (medium confidence, conservative).

- Reduced motion, accessibility (WCAG AA keyboard focus + contrast), and bilingual EN/ES strings are part of the front end but do not affect the math.

- To rebuild: implement the pools and streams in Section 2, the constants in Section 6, the per-facility tables in Section 7, the region tables in Section 8, the count/qualification logic in Section 4, and the escalation in Section 5. Everything else (animation, copy, layout) is presentation.

End of specification.